A7: Interrupt processing in x86 real mode [Practice assignment instructions]

A7:  Interrupt processing in x86 real mode

Open the COAL-A7 submission page and in the Feedback comments section find the task that has been assigned to you.

According to the task, create a timer interrupt handler procedure that outputs text and/or sound at specified time intervals.

It must be implemented as a 16-bit TSR program that works in x86 real mode (i.e., DOS).

The executable code is an EXE-format file that must be executed in the DOSBox emulation environment.

To output text to the console screen (text screen format – 25 lines, 80 columns, 16 colors), you must use the recording of symbols and their attributes in the video adapter memory (on page 0, starting from segment address B8000).

Channel 2 of the interval timer must be used to generate sound. The program does not need to check whether it is already in memory as a timer interrupt processing program.

The program is not allowed to use ready-made library functions for information conversion and output.

You may use the provided COAL2025 16 toolkit.

The MASM template code for the 16-bit moduleA7.asm:

.186

.model small

option casemap:none

.stack  100h

.code

; === Your interrupt handler procedure ===

int8h   proc far

;:::

int8h   endp

; --- Data used by the interrupt handler ---

oldint8 dw 0        ; saved entry point address of the previous handler

;:::

; Keep the code in memory up to this address

highaddr:

; === Startup code ===

main:               ; Entry point of the startup code

push    cs      ; Data is stored in the code segment

pop     ds      ; so copy cs to ds

mov     dx, offset msgInit

mov     ah, 9

int     21h

; Get and save the entry point address of the currently active ISR

mov     ax, 0

mov     es, ax                  ; segment 0

mov     bx, es:[4*8]            ; offset value from interrupt vector

mov     word ptr oldint8, bx    ; save offset value of the entry point address of the active ISR

mov     ax, es:[4*8+2]          ; segment value from interrupt vector

mov     word ptr oldint8+2, ax  ; save segment value of the entry point address of the active ISR

; Set new interrupt vector - entry point address of our handler int8h

mov     es:[4*8], offset int8h  ; entry point offset of the new interrupt handler

mov     ax, cs

mov     es:[4*8+2], ax          ; entry point segment of the new interrupt handler

; Show message that our interrupt handler is active

mov     dx, offset msgOK

mov     ah, 9

int     21h

; Exit and keep our handler code in memory

lea     dx, highaddr

add     dx, 10Fh    ; add size of the PSP and round up to the paragraph

shr     dx, 4       ; divide by 16

mov     ax, 3100h   ; function 31h, return code 00h

int     21h         ; return to the system and keep the code of our ISR in memory

; Data used by the startup code

msgInit DB '...',13,10,'$' ; update to reflect the assigned task description

msgOK   DB 'Interrupt Service Routine is activated',13,10,'$'

end main